BGP Documentation

Ian Kwan – CCNA

Purpose

In the modern network environment, different routing protocols such as OSPF, EIGRP, and IS-IS are used to route traffic over various autonomous systems (ASes). Each protocol runs on different metrics and algorithms causing incompatibility between networks running different routing protocols. BGP addresses this issue, offering an efficient and automatic method of route redistribution over multiple routing protocols. Thus, the purpose of this lab is to configure and implement BGP to facilitate the redistribution of routes between three different ASes running OSPF, EIGRP, and IS-IS respectively, establishing full connectivity from any point on the network.

Background

We will first discuss the different routing protocols used in this lab, followed by an in-depth analysis of BGP and its implementation.

Open Shortest Path First (OSPF) is an Internal Gateway Protocol (IGP). It uses the exchange of hello packets and Link-state updates (LSUs) to discover and keep neighbors and synchronize Link-state Databases (LDSBs). Link-state advertisements (LSAs) between routers help map out the topology of the autonomous system topology. But to reduce overhead, OSPF elects a designated router within an AS. The DR is elected based on many things but firstly the highest OSPF priority. The DR is the only router to establish full adjacency with other routers. Thus, all LSA traffic goes to the DR rather than each router, reducing overhead. OSPF uses the Shortest Path First algorithm to calculate the best path to every destination within the AS based on the LDSB. OSPF can also divide an AS into multiple areas to reduce the impact of link-state changes. Each area has its own independent LDSB, helping with scaling problems.

Enhanced Interior Gateway Protocol (EIGRP) is a Cisco proprietary IGP. It also uses the exchange of hello packets to discover and keep neighbors. Routers send update packets to neighbors so each neighbor can update and build an accurate topology table. This system of relying on updates from neighbors to the topology is called a distance-vector system. After building a table, using the Diffusing Update Algorithm (DUAL), the best paths, called successors, to each destination are calculated. Backup paths are also generated called feasible successors.

Intermediate System to Intermediate System Protocol (IS-IS) is another IGP. It is generally used for larger networks. Routers send hello packets to form adjacencies. There are two main types of IS-IS adjacencies: Level 1 and Level 2. Level 1 adjacencies are formed between routers within the same area. Level 2 adjacencies are formed between router in different areas. Level 2 adjacencies connect all the areas together within the same AS acting as the IS-IS backbone. Level 1-2 also exists. This is when a router boards two different areas and belongs to both areas. Thus, the have the capability to establish Level 1 and Level 2 adjacencies. Each area has their own LDSB, helping to divide the topology and reduces number of routers impacted link-state changes.

Border Gateway Protocol (BGP) is an inter-Autonomous System routing protocol also known as an External Gateway protocol (EGP). BGP is part of the category of protocols that facilitates the exchange of routing information and the detection of routing loops. The redistribution of routes is facilitated between ASes that use different routing protocols, which would otherwise be unable to route traffic to external destinations. EGPs are commonly described as the postal service of the modern network environment. ASes are described as independent cities and towns with hundreds of mailboxes which require the postal service to transport packets between cities.

BGP is also used to make external routing decisions. One example is calculating the most efficient routing path. A certain packet might have multiple paths across different ASes to reach a distant destination. BGP analyzes the hop counts and weights to determine the most efficient and preferred route. These two aspects of route selection are only a small part of the complex decision-making process of route selection.

BGP recognizes different ASes through AS numbers (ASNs). An ASN is used as an identifier that is made of 16 bits allowing for over 65 thousand combinations. Border routers of ASes have different ASNs which help them recognize and establish peer adjacencies.

BGP connections are established through peer sessions. Peer sessions are opened by ASes looking to establish adjacency after receiving information of a given BGP neighbor. ASes will establish a TCP/IP connection which facilitates the route redistribution that BGP is known for.

BGP’s latest version, BGP-4, predates to IPv6. Unlike newer protocols such as OSPF, which have two separate versions for IPv4 and IPv6 routing, BGP-4 can route both. This is done with the help of address family identifiers (AFIs). MP-BGP is an extension of BGP-4 which categorizes information such as the unicast and multicast routing further with the use of subsequent address family identifiers (SAFIs). BGP separates the two IP protocols using AFIs and tells their neighbors which protocol is being used by specifying a certain AFI+SAFI combination in the OPEN message at the start of each BGP sessions. But usually, routers will exchange IPv6 information over existing IPv4 BGP sessions rather than establish a brand-new session.

There are two modes of BGP: external BGP (eBGP) and internal BGP (iBGP). eBGP is used to exchange routing information between different ASes. eBGP is the mode used across the internet to route traffic between remote ASes. iBGP redistributes eBGP routing information within an AS and is usually used as a verification of proper redistribution of routes within an AS or between routers with the same ASN.

eBGP is the general mode of BGP used to exchange routing information and redistribute routes across the internet. eBGP routes have an administrative distance (AD) of 20. eBGP uses the time-to-live (TTL) metric to prevent packets from being propagated across the network forever. The default value of TTL for eBGP is 1. This is because eBGP assumes that peers are already directly connected. Thus, sessions will not extend over multiple ASes as they are usually only established through direct connection. eBGP also prevents loops through its AS_PATH attribute. When an eBGP router is sending out eBGP information it will add its ASN to the AS_PATH list so that the packet will be discarded if it is ever received by a router with the same ASN. This ensures protection against any unnecessary propagation of packets.

iBGP helps to confirm the consistency of data and eBGP-learned routes between BGP routers with the same ASN. This is especially the case when BGP routers are connected to different ASes of a network and need to verify that the redistribution of learned routes is properly propagated. Thus, iBGP routes have an AD of 200, as it is used more for verification rather than actual propagation of packets. The default value of TTL for iBGP is 255. BGP routers within the same AS are usually border routers which usually make these routers non-directly connected. To prevent loops the split horizon feature is used. This feature ensures that iBGP speakers, or routers propagating iBGP information, do not change the next-hop IP address of an eBGP route when advertising it to other routers. It also ensures that learned iBGP routes or prefixes don’t get advertised to other routers. This prevents unnecessary propagation of iBGP routes.

BGP uses route maps to control the flow of traffic that enters and exits an AS. It acts as a security gate that only allows wanted routes to enter and be redistributed within the AS and prevents unwanted advertisements of learned routes within the AS. Route maps can apply tags to certain routes so that only routes with matching tags are allowed through. Additionally, route-maps can act like ACL lists and only accept routes from permitted IPv4 or IPv6 ranges. This can be done by matching routes to set prefix-lists that specify which routes are permitted.

Additional features of route maps include the community and local preference features. The community feature lets routers further control traffic, not just through prefixes but specific groups. The community feature also allows routers to specify where a route is to be advertised, for example only locally. Local preference sets the preference of a route within an AS. The default value of local preference for routes is 100. But this value can be changed to a higher value so that routers will use that route over other routes with default local preference.

Configuration - Instructions

Topology

Topology – IPv4

| Device | G0/0/0 | G0/0/1 | Lo0 | Router-ID |
| --- | --- | --- | --- | --- |
| R1 |  | 192.168.1.1/24 | 10.10.1.1/24 | 1.1.1.1 |
| R2 | 192.168.1.2/24 | 192.168.2.1/24 | 10.10.2.1/24 | 2.2.2.2 |
| R3 | 192.168.2.2/24 | 192.168.3.1/24 | 10.10.3.1/24 | 3.3.3.3 |
| R4 | 192.168.3.2/24 | 192.168.4.1/24 | 10.10.4.1/24 | 4.4.4.4 |
| R5 | 192.168.4.2/24 | 192.168.5.1/24 | 10.10.5.1/24 | 49.0001.0000.0000.000a.00 |
| R6 | 192.168.5.2/24 |  | 10.10.6.1/24 | 49.0001.0000.0000.000b.00 |

Loopback interfaces are not necessary in establishing full connectivity across this topology, but it helps with verification of BGP. The loopback interfaces help make the BGP redistribution visible on the routing table.

Topology – IPv6

| Device | G0/0/0 | G0/0/1 | Lo0 | Router-ID |
| --- | --- | --- | --- | --- |
| R1 |  | 2001:db8:acad:1::1/64 | 2001:db8:acad:11::1/64 | 1.1.1.1 |
| R2 | 2001:db8:acad:1::2/64 | 2001:db8:acad:2::1/64 | 2001:db8:acad:12::1/64 | 2.2.2.2 |
| R3 | 2001:db8:acad:2::2/64 | 2001:db8:acad:3::1/64 | 2001:db8:acad:13::1/64 | 3.3.3.3 |
| R4 | 2001:db8:acad:3::2/64 | 2001:db8:acad:4::1/64 | 2001:db8:acad:14::1/64 | 4.4.4.4 |
| R5 | 2001:db8:acad:4::2/64 | 2001:db8:acad:5::1/64 | 2001:db8:acad:15::1/64 | 49.0001.0000.0000.000a.00 |
| R6 | 2001:db8:acad:5::2/64 |  | 2001:db8:acad:16::1/64 | 49.0001.0000.0000.000b.00 |

Configuration Instructions

OSPF

OSPFv2

| Command or Action | Purpose |
| --- | --- |
| router ospf 1 | Enables OSPFv2 on the router and sets process ID to 1. |
| router-id 1.1.1.1 | Sets the router-id to 1.1.1.1 which is used to identify the router from other routers in the OSPF network. |
| network 192.168.1.0 0.0.0.255 area 0 | Adds a connected network via ip address range and wildcard mask into OSPFv2 and adds it to area 0, the backbone area. |
| ip ospf 1 area 0 | Adds a connected network via a router interface into OSPFv2 and adds it to area 0, the backbone area. |
| clear ip ospf process | Clears and resets all OSPFv2 processes. |

Example Router Configuration:

Rx(config)#router ospf (process-ID)

Rx(config-router)#router-id (router-ID)

Rx(config-router)#exit

Rx(config)#int g0/0/x

Rx(config-if)#ip address (IPv4 address) (subnet mask)

Rx(config-if)#ip ospf (process-ID) area (area)

Rx(config-if)#no shutdown

Rx(config-if)#exit

Rx(config)#int lo0

Rx(config-if)#ip address (IPv4 address) (subnet mask)

Rx(config-if)#ip ospf (process-ID) area (area)

Rx(config-if)#no shutdown

Rx(config-if)#end

Rx#clear ip ospf process

Reset selected OPSFv2 processes? [no]:yes

OSPFv3

| Command or Action | Purpose |
| --- | --- |
| router ospfv3 1 | Enables OSPFv3 on the router and sets process ID to 1. |
| router-id 1.1.1.1 | Sets the router-id to 1.1.1.1 which is used to identify the router from other routers in the OSPF network. |
| ip ospf 1 area 0 | Adds a connected network via a router interface into OSPFv3 and adds it to area 0, the backbone area. |
| clear ip ospfv3 process | Clears and resets all OSPFv3 processes. |

Example Router Configuration:

Rx(config)#ipv6 router ospf (process-ID)

Rx(config-router)#router-id (router-ID)

Rx(config-router)#exit

Rx(config)#int g0/0/x

Rx(config-if)#ipv6 address (IPv6 address)

Rx(config-if)#ipv6 ospf (process-ID) area (area)

Rx(config-if)#no shutdown

Rx(config-if)#exit

Rx(config)#int lo0

Rx(config-if)#ipv6 address (IPv6 address)

Rx(config-if)#ip ospf (process-ID) area (area)

Rx(config-if)#no shutdown

Rx(config-if)#end

Rx#clear ipv6 ospf process

Reset selected OPSFv3 processes? [no]:yes

EIGRP

EIGRPv4

| Command or Action | Purpose |
| --- | --- |
| ip router eigrp 100 | Enables EIGRPv4 on the router and sets process ID to 100. |
| eigrp router-id 1.1.1.1 | Sets router ID to 1.1.1.1 which is used to identify the router from other routers in the EIGRP network. |
| network 192.168.3.0 0.0.0.255 | Adds a connected network via network range and wildcard mask into EIGRPv4 database. |
| default-metric 10000 100 255 1 1500 | Sets the default metrics to the inputted configuration. |

Example Router Configuration:

Rx(config)#ip router eigrp (process-ID)

Rx(config-router)#eigrp router-id (router-ID)

Rx(config-router)#default-metric (metrics)

Rx(config-router)#network (IPv4 address) (Wildcard mask)

Rx(config-router)#network (IPv4 address) (Wildcard mask)

Rx(config-router)#exit

Rx(config-if)#end

EIGRPv6

| Command or Action | Purpose |
| --- | --- |
| ipv6 router eigrp 100 | Enables EIGRPv4 on the router and sets process ID to 100. |
| eigrp router-id 1.1.1.1 | Sets router ID to 1.1.1.1 which is used to identify the router from other routers in the EIGRP network. |
| ipv6 enable | Enables ipv6 on an interface (not necessary if ipv6 unicast routing is already enabled). |
| ipv6 eigrp 100 | Adds a connected network via an interface into EIGRPv6 database. |
| default-metric 10000 100 255 1 1500 | Sets the default metrics to the inputted configuration. |

Example Router Configuration:

Rx(config)#ipv6 router eigrp (process-ID)

Rx(config-router)#eigrp router-id (router-ID)

Rx(config-router)#default-metric (metrics)

Rx(config-router)#exit

Rx(config)#int g0/0/x

Rx(config-if)#ipv6 address (IPv6 address)

Rx(config-if)#ipv6 enable

Rx(config-if)#ipv6 eigrp (process-ID)

Rx(config-if)#no shutdown

Rx(config-if)#exit

Rx(config)#int lo0

Rx(config-if)#ipv6 address (IPv6 address)

Rx(config-if)#ipv6 enable

Rx(config-if)#ipv6 eigrp (process-ID)

Rx(config-if)#no shutdown

Rx(config-if)#end

IS-IS

IS-IS (IPv4 Address Family)

| Command or Action | Purpose |
| --- | --- |
| router isis 1 | Enables IS-IS on the router with process ID 1. |
| net 49.0001.0000.0000.000b.00 | Sets the router net ID, the IS-IS identifier, to 49.001.0000.0000.000b.00 |
| ip router isis 1 | Adds a connected network via an interface in the IS-IS database |

Example Router Configuration:

Rx(config)#router isis (process-ID)

Rx(config-router)#net (NET address)

Rx(config-router)#exit

Rx(config)#int g0/0/x

Rx(config-if)#ip address (IPv4 address) (subnet mask)

Rx(config-if)#ip router isis (process-ID)

Rx(config-if)#no shutdown

Rx(config-if)#exit

Rx(config)#int lo0

Rx(config-if)#ip address (IPv4 address) (subnet mask)

Rx(config-if)#ip router isis (process-ID)

Rx(config-if)#no shutdown

Rx(config-if)#end

IS-IS (IPv6 Address Family)

| Command or Action | Purpose |
| --- | --- |
| router isis 1 | Enables IS-IS on the router with process ID 1. |
| net 49.0001.0000.0000.000b.00 | Sets the router net ID, the IS-IS identifier, to 49.001.0000.0000.000b.00 |
| Ipv6 router isis 1 | Adds a connected network via an interface in the IS-IS database |

Example Router Configuration:

Rx(config)#router isis (process-ID)

Rx(config-router)#net (NET address)

Rx(config-router)#exit

Rx(config)#int g0/0/x

Rx(config-if)#ipv6 address (IPv6 address)

Rx(config-if)#ipv6 enable

Rx(config-if)#ipv6 router isis (process-ID)

Rx(config-if)#no shutdown

Rx(config-if)#exit

Rx(config)#int lo0

Rx(config-if)#ipv6 address (IPv6 address)

Rx(config-if)#ipv6 enable

Rx(config-if)#ipv6 router isis (process-ID)

Rx(config-if)#no shutdown

Rx(config-if)#end

BGP – Neighbor Adjacencies & Redistribution

(IPv4 Address Family)

| Command or Action | Purpose |
| --- | --- |
| router bgp 65001 | Enables BGP on the router and sets the ASN to 65001. |
| address-family ipv4 | Enters the IPv4 address family of BGP. |
| neighbor 192.168.2.2 remote-as 65002 | Adds a neighbor AS border router via IPv4 address. |
| neighbor 192.168.2.2 activate | Enables the router to actively seek to establish adjacency. |
| redistribute bgp 65001 include-connected | Redistributes BGP routes from ASN 65001 & directly connected routes with default metrics. |
| redistribute ospf 1 include-connected | Redistributes OSPF routes under process ID 1 & directly connected routes with default metrics. |
| redistribute eigrp 100 include-connected | Redistributes EIGRP routes under process ID 100 & directly connected routes with default metrics. |
| redistribute isis 1 level-1 include-connected | Redistributes IS-IS level-1 routes under process ID 1 & directly connected routes with default metrics. |

BGP (IPv4: OSPF to BGP)

Example Router Configuration:

Rx(config)#router bgp (ASN)

Rx(config-router)#address-family ipv4

Rx(config-router-ad)#neighbor (IPv4 address) remote-as (ASN)

Rx(config-router-ad)#neighbor (IPv4 address) activate

Rx(config-router-ad)#redistribute (IGP Protocol) (process-ID) include-connected

Rx(config-router-ad)#end

OSPF

Rx(config)#router ospf (Process-ID)

Rx(config-router)#redistribute bgp (ASN) include-connected

Rx(config-router)#end

IS-IS

Rx(config)#router isis (Process-ID)

Rx(config-router)#address-family ipv4

Rx(config-router-ad)#redistribute bgp (ASN) include-connected

Rx(config-router-ad)#end

EIGRP

Rx(config)#router eigrp (Process-ID)

Rx(config-router)#redistribute bgp (ASN) include-connected metric (bandwidth, delay, reliability, load, MTU)

Rx(config-router)#end

(IPv4 Address Family)

| Command or Action | Purpose |
| --- | --- |
| router bgp 65001 | Enables BGP on the router and sets the ASN to 65001. |
| address-family ipv6 | Enters the IPv6 address family. |
| neighbor 2001:db8:acad:2::2 remote-as 65002 | Adds a neighbor AS border router via IPv6 address. |
| neighbor 2001:db8:acad:2::2 activate | Enables the router to actively seek to establish adjacency. |
| redistribute bgp 65001 include-connected metric 10000 100 255 1 1500 | Redistributes BGP routes from ASN 65001 & directly connected routes with default metrics. |
| redistribute ospfv3 1 include-connected metric 10000 100 255 1 1500 | Redistributes OSPF routes under process ID 1 & directly connected routes with default metrics. |
| redistribute eigrp 100 include-connected metric 10000 100 255 1 1500 | Redistributes EIGRP routes under process ID 100 & directly connected routes with default metrics. |
| redistribute isis 1 level-1 include-connected metric 10000 100 255 1 1500 | Redistributes IS-IS level-1 routes under process ID 1 & directly connected routes with default metrics. |

BGP (Address Family IPv6)

Example Router Configuration:

Rx(config)#router bgp (ASN)

Rx(config-router)#address-family ipv6

Rx(config-router-ad)#neighbor (IPv6 address) remote-as (ASN)

Rx(config-router-ad)#neighbor (IPv6 address) activate

Rx(config-router-ad)#redistribute (IGP Protocol) (Process-ID) include-connected

Rx(config-router-ad)#end

OSPF

Rx(config)#router ospfv3 (Process-ID)

Rx(config-router)#redistribute bgp (ASN) include-connected

Rx(config-router)#end

IS-IS

Rx(config)#router isis (Process-ID)

Rx(config-router)#address-family ipv6

Rx(config-router-ad)#redistribute bgp (ASN) include-connected

Rx(config-router-ad)#end

EIGRP

Rx(config)#ipv6 router eigrp (Process-ID)

Rx(config-router)#redistribute bgp (ASN) include-connected metric (bandwidth, delay, reliability, load, MTU)

Rx(config-router)#end

Additional Setup

Weights

| Command or Action | Purpose |
| --- | --- |
| neighbor 192.168.2.2 weight 500 | Sets the weight of BGP neighbor 192.168.2.2 to 500. |

Example Router Configuration:

Rx(config)#router bgp (ASN)

Rx(config-router)#address-family ipv4

Rx(config-router-ad)# neighbor (neighbor IPv4 address) weight (weight)

Rx(config-router-ad)#end

Rx(config)#router bgp (ASN)

Rx(config-router)#address-family ipv6

Rx(config-router-ad)# neighbor (neighbor IPv6 address) weight (weight)

Rx(config-router-ad)#end

R2 Configuration:

router bgp 65001

bgp log-neighbor-changes

no bgp default ipv4-unicast

neighbor 2001:DB8:ACAD:2::2 remote-as 65002

neighbor 192.168.2.2 remote-as 65002

!

address-family ipv4

neighbor 192.168.2.2 activate

neighbor 192.168.2.2 weight 500

exit-address-family

!

address-family ipv6

neighbor 2001:DB8:ACAD:2::2 activate

neighbor 2001:DB8:ACAD:2::2 weight 500

exit-address-family

Route-Maps

| Command or Action | Purpose |
| --- | --- |
| ip prefix-list OSPF-IMPORT seq 5 permit 10.10.3.0/24 | Configures prefix-list OSPF-IMPORT to allow routes to the 10.10.3.0/24 network. |
| route-map BGP-TO-OSPFv4 permit 10 | Opens the route map BGP-TO-OSPFv4 up to be defined. |
| match ip address prefix-list OSPF-IMPORT | Defines the route map to permit according to IP addresses matching the prefix list OSPF-IMPORT. |
| neighbor 192.168.2.2 route-map BGP-TO-OSPFv4 in | Applies the route map to inbound traffic from neighbor 192.168.2.2 on BGP. |

There are many other uses to route maps, but the most simple way is to filter traffic based on IP addresses. In the R2 example configuration there are other route map configurations that can be experimented with.

Example Router Configuration:

Rx(config)#ip prefix-list (prefix-list) seq (priority) (permit or deny) (IPv4 address range)

Rx(config)#route-map (route map)(permit or deny) (priority)

Rx(config-route-map)#match ip address prefix-list (prefix-list)

Rx(config-route-map)#exit

Rx(config)#router bgp (ASN)

Rx(config-router)#address-family ipv4

Rx(config-router-ad)# neighbor (neighbor IPv4 address) route-map (route-map) (inbound or outbound)

Rx(config-router-ad)#end

Rx(config)#ip prefix-list (prefix-list) seq (priority) (permit or deny) (IPv4 address range)

Rx(config)#route-map (route map)(permit or deny) (priority)

Rx(config-route-map)#match ipv6 address prefix-list (prefix-list)

Rx(config-route-map)#exit

Rx(config)#router bgp (ASN)

Rx(config-router)#address-family ipv6

Rx(config-router-ad)# neighbor (neighbor IPv6 address) route-map (route-map) (inbound or outbound)

Rx(config-router-ad)#end

R2 Configuration:

router bgp 65001

bgp log-neighbor-changes

no bgp default ipv4-unicast

neighbor 2001:DB8:ACAD:2::2 remote-as 65002

neighbor 192.168.2.2 remote-as 65002

address-family ipv4

neighbor 192.168.2.2 activate

neighbor 192.168.2.2 route-map BGP-TO-OSPFv4 in

exit-address-family

address-family ipv6

neighbor 2001:DB8:ACAD:2::2 activate

neighbor 2001:DB8:ACAD:2::2 route-map BGP-TO-OSPFv6 in

exit-address-family

ip prefix-list OSPF-IMPORT seq 5 permit 10.10.3.0/24

ip prefix-list OSPF-IMPORT seq 10 permit 10.10.4.0/24

ip prefix-list OSPF-IMPORT seq 15 permit 10.10.5.0/24

ip prefix-list OSPF-IMPORT seq 20 permit 10.10.6.0/24

ip prefix-list OSPF-IMPORT seq 25 permit 192.168.2.0/24

ip prefix-list OSPF-IMPORT seq 30 permit 192.168.3.0/24

ip prefix-list OSPF-IMPORT seq 35 permit 192.168.4.0/24

ip prefix-list OSPF-IMPORT seq 40 permit 192.168.5.0/24

ipv6 prefix-list OSPF-IMPORT seq 5 permit 2001:DB8:ACAD:2::/64

ipv6 prefix-list OSPF-IMPORT seq 10 permit 2001:DB8:ACAD:3::/64

ipv6 prefix-list OSPF-IMPORT seq 15 permit 2001:DB8:ACAD:4::/64

ipv6 prefix-list OSPF-IMPORT seq 20 permit 2001:DB8:ACAD:5::/64

ipv6 prefix-list OSPF-IMPORT seq 25 permit 2001:DB8:ACAD:13::/64

ipv6 prefix-list OSPF-IMPORT seq 30 permit 2001:DB8:ACAD:14::/64

ipv6 prefix-list OSPF-IMPORT seq 35 permit 2001:DB8:ACAD:15::/64

ipv6 prefix-list OSPF-IMPORT seq 40 permit 2001:DB8:ACAD:16::/64

route-map BGP-TO-OSPFv6 permit 10

match ipv6 address prefix-list OSPF-IMPORT

set metric 10

set tag 100

set local-preference 120

route-map BPG-TO-OSPFv6 permit 10

route-map BGP-TO-OSPFv4 permit 10

match ip address prefix-list OSPF-IMPORT

set metric 10

set tag 100

set local-preference 120

route-map BPG-TO-OSPFv4 permit 10

Network Configuration

R1

hostname R1

no ip domain lookup

interface Loopback0

ip address 10.10.1.1 255.255.255.0

ip ospf 1 area 2

ipv6 address 2001:DB8:ACAD:11::1/64

ipv6 ospf 1 area 2

interface GigabitEthernet0/0/1

ip address 192.168.1.1 255.255.255.0

ip ospf 1 area 0

negotiation auto

ipv6 address FE80::1 link-local

ipv6 address 2001:DB8:ACAD:1::1/64

ipv6 ospf 1 area 0

router ospfv3 1

router-id 1.1.1.1

router ospf 1

router-id 1.1.1.1

network 192.168.1.0 0.0.0.255 area 0

end

R2

hostname R2

no ip domain lookup

ipv6 unicast-routing

interface Loopback0

ip address 10.10.2.1 255.255.255.0

ip ospf 1 area 1

ipv6 address 2001:DB8:ACAD:12::1/64

ipv6 ospf 1 area 1

interface GigabitEthernet0/0/0

ip address 192.168.1.2 255.255.255.0

ip ospf 1 area 0

ipv6 address FE80::2 link-local

ipv6 address 2001:DB8:ACAD:1::2/64

ipv6 ospf 1 area 0

interface GigabitEthernet0/0/1

ip address 192.168.2.1 255.255.255.0

ipv6 address FE80::1 link-local

ipv6 address 2001:DB8:ACAD:2::1/64

router ospfv3 1

router-id 2.2.2.2

router ospf 1

router-id 2.2.2.2

redistribute connected subnets

redistribute bgp 65001 subnets

network 192.168.1.0 0.0.0.255 area 0

router bgp 65001

bgp log-neighbor-changes

no bgp default ipv4-unicast

neighbor 2001:DB8:ACAD:2::2 remote-as 65002

neighbor 192.168.2.2 remote-as 65002

address-family ipv4

redistribute connected

redistribute ospf 1 include-connected

neighbor 192.168.2.2 activate

exit-address-family

address-family ipv6

redistribute connected

redistribute ospfv3 1 include-connected

neighbor 2001:DB8:ACAD:2::2 activate

exit-address-family

end

R3

hostname R3

no ip domain lookup

ipv6 unicast-routing

interface Loopback0

ip address 10.10.3.1 255.255.255.0

ipv6 address 2001:DB8:ACAD:13::1/64

ipv6 enable

ipv6 eigrp 100

interface GigabitEthernet0/0/0

ip address 192.168.2.2 255.255.255.0

ipv6 address FE80::2 link-local

ipv6 address 2001:DB8:ACAD:2::2/64

interface GigabitEthernet0/0/1

ip address 192.168.3.1 255.255.255.0

ipv6 address FE80::1 link-local

ipv6 address 2001:DB8:ACAD:3::1/64

ipv6 enable

ipv6 eigrp 100

router eigrp 100

default-metric 10000 100 255 1 1500

network 10.0.0.0

network 192.168.3.0

redistribute bgp 65002 metric 10000 100 255 1 1500

redistribute ospf 1

redistribute connected metric 10000 100 255 1 1500

eigrp router-id 3.3.3.3

router bgp 65002

bgp log-neighbor-changes

no bgp default ipv4-unicast

neighbor 2001:DB8:ACAD:2::1 remote-as 65001

neighbor 192.168.2.1 remote-as 65001

neighbor 192.168.4.2 remote-as 65003

address-family ipv4

redistribute eigrp 100

neighbor 192.168.2.1 activate

neighbor 192.168.4.2 activate

exit-address-family

address-family ipv6

redistribute eigrp 100 include-connected

neighbor 2001:DB8:ACAD:2::1 activate

exit-address-family

ipv6 router eigrp 100

eigrp router-id 3.3.3.3

redistribute bgp 65002 metric 10000 100 255 1 1500

redistribute connected

end

R4

hostname R4

no ip domain lookup

ipv6 unicast-routing

interface Loopback0

ip address 10.10.4.1 255.255.255.0

ipv6 address 2001:DB8:ACAD:14::1/64

ipv6 enable

ipv6 eigrp 100

interface GigabitEthernet0/0/0

ip address 192.168.3.2 255.255.255.0

ipv6 address FE80::2 link-local

ipv6 address 2001:DB8:ACAD:3::2/64

ipv6 enable

ipv6 eigrp 100

interface GigabitEthernet0/0/1

ip address 192.168.4.1 255.255.255.0

ipv6 address FE80::1 link-local

ipv6 address 2001:DB8:ACAD:4::1/64

router eigrp 100

network 10.0.0.0

network 192.168.3.0

network 192.168.4.0

redistribute bgp 65002 metric 100000 100 255 1 1500

redistribute connected metric 10000 100 255 1 1500

eigrp router-id 4.4.4.4

router bgp 65002

bgp log-neighbor-changes

no bgp default ipv4-unicast

neighbor 2001:DB8:ACAD:2::1 remote-as 65001

neighbor 2001:DB8:ACAD:4::2 remote-as 65003

neighbor 192.168.2.1 remote-as 65001

neighbor 192.168.4.2 remote-as 65003

address-family ipv4

redistribute connected

redistribute eigrp 100

neighbor 192.168.2.1 activate

neighbor 192.168.4.2 activate

exit-address-family

address-family ipv6

redistribute connected

redistribute eigrp 100 include-connected

neighbor 2001:DB8:ACAD:2::1 activate

neighbor 2001:DB8:ACAD:4::2 activate

exit-address-family

ipv6 router eigrp 100

eigrp router-id 4.4.4.4

redistribute bgp 65002 metric 10000 100 255 1 1500

redistribute connected metric 10000 100 255 1 1500

end

R5

hostname R5

no ip domain lookup

ipv6 unicast-routing

interface Loopback0

ip address 10.10.5.1 255.255.255.0

ip router isis 1

ipv6 address 2001:DB8:ACAD:15::1/64

ipv6 router isis 1

interface GigabitEthernet0/0/0

ip address 192.168.4.2 255.255.255.0

ipv6 address FE80::2 link-local

ipv6 address 2001:DB8:ACAD:4::2/64

interface GigabitEthernet0/0/1

ip address 192.168.5.1 255.255.255.0

ip router isis 1

ipv6 address FE80::1 link-local

ipv6 address 2001:DB8:ACAD:5::1/64

ipv6 router isis 1

router isis 1

net 49.0001.0000.0000.000a.00

address-family ipv4

redistribute connected

redistribute bgp 65003

exit-address-family

address-family ipv6

redistribute connected

redistribute bgp 65003

exit-address-family

router bgp 65003

bgp log-neighbor-changes

neighbor 2001:DB8:ACAD:4::1 remote-as 65002

neighbor 192.168.4.1 remote-as 65002

address-family ipv4

redistribute connected

redistribute isis 1 level-1

no neighbor 2001:DB8:ACAD:4::1 activate

neighbor 192.168.4.1 activate

exit-address-family

address-family ipv6

redistribute isis 1 level-1-2 include-connected

redistribute isis level-1

neighbor 2001:DB8:ACAD:4::1 activate

exit-address-family

end

R6

hostname R6

no aaa new-model

no ip domain lookup

ipv6 unicast-routing

interface Loopback0

ip address 10.10.6.1 255.255.255.0

ip router isis 1

ipv6 address 2001:DB8:ACAD:16::1/64

ipv6 router isis 1

interface GigabitEthernet0/0/0

ip address 192.168.5.2 255.255.255.0

ip router isis 1

ipv6 address FE80::2 link-local

ipv6 address 2001:DB8:ACAD:5::2/64

ipv6 router isis 1

router isis 1

net 49.0001.0000.0000.000b.00

end

Proof of Connectivity, BGP, EIGRP, IS-IS, and OSPF

Proof of Connectivity

Pings across network:

IPv4: R1 to R6 and R6 to R1

R1#ping 10.10.6.1

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 10.10.6.1, timeout is 2 seconds:

!!!!!

Success rate is 100 percent (5/5), round-trip min/avg/max = 1/3/13 ms

R1#traceroute 10.10.6.1

Type escape sequence to abort.

Tracing the route to 10.10.6.1

VRF info: (vrf in name/id, vrf out name/id)

1 192.168.1.2 1 msec 1 msec 1 msec

2 192.168.2.2 1 msec 0 msec 1 msec

3 192.168.3.2 1 msec 1 msec 1 msec

4 192.168.4.2 1 msec 1 msec 1 msec

5 192.168.5.2 2 msec 2 msec *

R6#ping 10.10.1.1

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 10.10.1.1, timeout is 2 seconds:

!!!!!

Success rate is 100 percent (5/5), round-trip min/avg/max = 1/1/2 ms

R6#traceroute 10.10.1.1

Type escape sequence to abort.

Tracing the route to 10.10.1.1

VRF info: (vrf in name/id, vrf out name/id)

1 192.168.5.1 1 msec 0 msec 1 msec

2 192.168.4.1 1 msec 1 msec 1 msec

3 192.168.3.1 0 msec 1 msec 1 msec

4 192.168.2.1 1 msec 2 msec 1 msec

5 192.168.1.1 1 msec *  1 msec

IPv6: R1 to R6 and R6 to R1

R1#ping 2001:db8:acad:16::1

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 2001:DB8:ACAD:16::1, timeout is 2 seconds:

!!!!!

Success rate is 100 percent (5/5), round-trip min/avg/max = 1/2/6 ms

R1#traceroute 2001:db8:acad:16::1

Type escape sequence to abort.

Tracing the route to 2001:DB8:ACAD:16::1

1 2001:DB8:ACAD:1::2 2 msec 1 msec 0 msec

2 2001:DB8:ACAD:2::2 0 msec 1 msec 1 msec

3 2001:DB8:ACAD:3::2 1 msec 2 msec 1 msec

4 2001:DB8:ACAD:4::2 1 msec 2 msec 1 msec

5 2001:DB8:ACAD:5::2 1 msec 2 msec 1 msec

R6#ping 2001:db8:acad:11::1

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 2001:DB8:ACAD:11::1, timeout is 2 seconds:

!!!!!

Success rate is 100 percent (5/5), round-trip min/avg/max = 1/2/6 ms

R6#traceroute 2001:db8:acad:11::1

Type escape sequence to abort.

Tracing the route to 2001:DB8:ACAD:11::1

1 2001:DB8:ACAD:5::1 1 msec 0 msec 0 msec

2 2001:DB8:ACAD:4::1 1 msec 1 msec 1 msec

3 2001:DB8:ACAD:3::1 1 msec 1 msec 1 msec

4 2001:DB8:ACAD:2::1 1 msec 2 msec 1 msec

5 2001:DB8:ACAD:1::1 1 msec 1 msec 2 msec

Connectivity is proven as packets can be sent across the whole network, between the different ASes. Not only that, because we pinged from both sides, we prove that the ASes have routes both ways through the network. We pinged over both IPv4 and IPv6 proving that we have BGP for both the IPv4 and IPv6 address family fully set up.

Proof of BGP

IPv4: BGP Database

R3#show ip bgp

BGP table version is 67, local router ID is 192.168.2.2

Status codes: s suppressed, d damped, h history, * valid, > best, i - internal,

r RIB-failure, S Stale, m multipath, b backup-path, f RT-Filter,

x best-external, a additional-path, c RIB-compressed,

Origin codes: i - IGP, e - EGP, ? - incomplete

RPKI validation codes: V valid, I invalid, N Not found

Network          Next Hop            Metric LocPrf Weight Path

*>  10.10.1.1/32     192.168.2.1              2             0 65001 ?

*>  10.10.2.0/24     192.168.2.1              0             0 65001 ?

*>  10.10.3.0/24     0.0.0.0                  0         32768 ?

*>  10.10.4.0/24     192.168.3.2         130816         32768 ?

*>  10.10.5.0/24     192.168.3.2          51456         32768 ?

*>  10.10.6.0/24     192.168.3.2          51456         32768 ?

*>  192.168.1.0      192.168.2.1              0             0 65001 ?

r>  192.168.2.0      192.168.2.1              0             0 65001 i

*>  192.168.3.0      0.0.0.0                  0         32768 ?

*>  192.168.4.0      192.168.3.2           3072         32768 ?

*>  192.168.5.0      192.168.3.2          51456         32768 ?

IPv4: BGP Routes

R4#show ip route bgp

Codes: L - local, C - connected, S - static, R - RIP, M - mobile, B - BGP

D - EIGRP, EX - EIGRP external, O - OSPF, IA - OSPF inter area

N1 - OSPF NSSA external type 1, N2 - OSPF NSSA external type 2

E1 - OSPF external type 1, E2 - OSPF external type 2, m - OMP

n - NAT, Ni - NAT inside, No - NAT outside, Nd - NAT DIA

i - IS-IS, su - IS-IS summary, L1 - IS-IS level-1, L2 - IS-IS level-2

ia - IS-IS inter area, * - candidate default, U - per-user static route

H - NHRP, G - NHRP registered, g - NHRP registration summary

o - ODR, P - periodic downloaded static route, l - LISP

a - application route

+ - replicated route, % - next hop override, p - overrides from PfR

& - replicated local route overrides by connected

Gateway of last resort is not set

10.0.0.0/8 is variably subnetted, 7 subnets, 2 masks

B        10.10.5.0/24 [20/0] via 192.168.4.2, 1d00h

B        10.10.6.0/24 [20/20] via 192.168.4.2, 1d00h

B     192.168.5.0/24 [20/0] via 192.168.4.2, 1d00h

IPv6: BGP Database

R3#show bgp ipv6 unicast

BGP table version is 503, local router ID is 192.168.2.2

Status codes: s suppressed, d damped, h history, * valid, > best, i - internal,

r RIB-failure, S Stale, m multipath, b backup-path, f RT-Filter,

x best-external, a additional-path, c RIB-compressed,

Origin codes: i - IGP, e - EGP, ? - incomplete

RPKI validation codes: V valid, I invalid, N Not found

Network          Next Hop            Metric LocPrf Weight Path

*>  2001:DB8:ACAD:1::/64

2001:DB8:ACAD:2::1

0             0 65001 ?

r>  2001:DB8:ACAD:2::/64

2001:DB8:ACAD:2::1

0             0 65001 ?

*>  2001:DB8:ACAD:3::/64

::                       0         32768 ?

*>  2001:DB8:ACAD:4::/64

FE80::2             281856         32768 ?

*>  2001:DB8:ACAD:5::/64

FE80::2             281856         32768 ?

*>  2001:DB8:ACAD:11::1/128

2001:DB8:ACAD:2::1

Network          Next Hop            Metric LocPrf Weight Path

1             0 65001 ?

*>  2001:DB8:ACAD:12::/64

2001:DB8:ACAD:2::1

0             0 65001 ?

*>  2001:DB8:ACAD:13::/64

::                       0         32768 ?

*>  2001:DB8:ACAD:14::/64

FE80::2             130816         32768 ?

*>  2001:DB8:ACAD:15::/64

FE80::2             281856         32768 ?

*>  2001:DB8:ACAD:16::/64

FE80::2             281856         32768 ?

IPv6: BGP Routes

R4#show ipv6 route bgp

IPv6 Routing Table - default - 15 entries

Codes: C - Connected, L - Local, S - Static, U - Per-user Static route

B - BGP, R - RIP, H - NHRP, I1 - ISIS L1

I2 - ISIS L2, IA - ISIS interarea, IS - ISIS summary, D - EIGRP

EX - EIGRP external, ND - ND Default, NDp - ND Prefix, DCE - Destination

NDr - Redirect, O - OSPF Intra, OI - OSPF Inter, OE1 - OSPF ext 1

OE2 - OSPF ext 2, ON1 - OSPF NSSA ext 1, ON2 - OSPF NSSA ext 2

a - Application, m - OMP

B   2001:DB8:ACAD:5::/64 [20/0], tag 65003

via FE80::2, GigabitEthernet0/0/1

B   2001:DB8:ACAD:15::/64 [20/0], tag 65003

via FE80::2, GigabitEthernet0/0/1

B   2001:DB8:ACAD:16::/64 [20/20], tag 65003

via FE80::2, GigabitEthernet0/0/1

We have proven that BGP is functional in two ways: through the BGP database and router routing tables. Showing the BGP database proves that the BGP is fully functional. The BGP database for both IPv4 and IPv6 have all network IP addresses documented showing that all routes from the different ASes have been properly redistributed into the BGP database. Not only that, we have also proven that R4 has learned 3 IPv4 BGP routes and 3 IPv6 BGP routes by showing the routing table. Therefore, we know that the BGP database is fully completed and that BGP routes are actively being used by routers within the network. Therefore, by showing that the BGP database is complete and fully functional and there are BGP routes used by routers within the network for both IPv4 and IPv6 we have proven that BGP has been fully and properly configured.

Proof of EIGRP

IPv4: EIGRP Neighbors

R4#show ip eigrp neighbors

EIGRP-IPv4 Neighbors for AS(100)

H   Address                 Interface              Hold Uptime   SRTT   RTO  Q  Seq

(sec)         (ms)       Cnt Num

0   192.168.3.1             Gi0/0/0                  11 1w0d        1   100  0  51

IPv4: EIGRP routes

R4#show ip route eigrp

Codes: L - local, C - connected, S - static, R - RIP, M - mobile, B - BGP

D - EIGRP, EX - EIGRP external, O - OSPF, IA - OSPF inter area

N1 - OSPF NSSA external type 1, N2 - OSPF NSSA external type 2

E1 - OSPF external type 1, E2 - OSPF external type 2, m - OMP

n - NAT, Ni - NAT inside, No - NAT outside, Nd - NAT DIA

i - IS-IS, su - IS-IS summary, L1 - IS-IS level-1, L2 - IS-IS level-2

ia - IS-IS inter area, * - candidate default, U - per-user static route

H - NHRP, G - NHRP registered, g - NHRP registration summary

o - ODR, P - periodic downloaded static route, l - LISP

a - application route

+ - replicated route, % - next hop override, p - overrides from PfR

& - replicated local route overrides by connected

Gateway of last resort is not set

10.0.0.0/8 is variably subnetted, 7 subnets, 2 masks

D EX     10.10.1.1/32

[170/281856] via 192.168.3.1, 00:20:36, GigabitEthernet0/0/0

D EX     10.10.2.0/24

[170/281856] via 192.168.3.1, 4d01h, GigabitEthernet0/0/0

D        10.10.3.0/24 [90/130816] via 192.168.3.1, 4d02h, GigabitEthernet0/0/0

D EX  192.168.1.0/24

[170/281856] via 192.168.3.1, 00:21:07, GigabitEthernet0/0/0

D EX  192.168.2.0/24 [170/281856] via 192.168.3.1, 4d01h, GigabitEthernet0/0/0

IPv6: EIGRP Neighbors

R4#show ipv6 eigrp neighbors

EIGRP-IPv6 Neighbors for AS(100)

H   Address                 Interface              Hold Uptime   SRTT   RTO  Q  Seq

(sec)         (ms)       Cnt Num

0   Link-local address:     Gi0/0/0                  13 1d00h       1   100  0  17

FE80::1

IPv6: EIGRP routes

R4#show ipv6 route eigrp

IPv6 Routing Table - default - 15 entries

Codes: C - Connected, L - Local, S - Static, U - Per-user Static route

B - BGP, R - RIP, H - NHRP, I1 - ISIS L1

I2 - ISIS L2, IA - ISIS interarea, IS - ISIS summary, D - EIGRP

EX - EIGRP external, ND - ND Default, NDp - ND Prefix, DCE - Destination

NDr - Redirect, O - OSPF Intra, OI - OSPF Inter, OE1 - OSPF ext 1

OE2 - OSPF ext 2, ON1 - OSPF NSSA ext 1, ON2 - OSPF NSSA ext 2

a - Application, m - OMP

EX  2001:DB8:ACAD:1::/64 [170/281856]

via FE80::1, GigabitEthernet0/0/0

EX  2001:DB8:ACAD:2::/64 [170/3072]

via FE80::1, GigabitEthernet0/0/0

EX  2001:DB8:ACAD:11::1/128 [170/281856]

via FE80::1, GigabitEthernet0/0/0

EX  2001:DB8:ACAD:12::/64 [170/281856]

via FE80::1, GigabitEthernet0/0/0

D   2001:DB8:ACAD:13::/64 [90/130816]

via FE80::1, GigabitEthernet0/0/0

We have proven that EIGRP is fully functional in two different ways: through showing EIGRP neighbors and routes. The EIGRP routing tables have both internal and external EIGRP routes showing that all the BGP routes have been properly redistributed and learned by the EIGRP AS. Not only that, EIGRP is also shown to be fully functional as the EIGRP neighbor database shows an EIGRP neighbor. Therefore, we have proven that EIGRP has been properly configured for both IPv4 and IPv6.

Proof of IS-IS

IS-IS: CLNS neighbors

R5#show clns neighbors

Tag 1:

System Id       Interface     SNPA                State  Holdtime  Type Protocol

R6              Gi0/0/1       0027.90d5.fa40      Up     28        L1L2 IS-IS

R6#show clns neighbors

Tag 1:

System Id       Interface     SNPA                State  Holdtime  Type Protocol

R5              Gi0/0/0       0027.90d5.fad1      Up     9         L1L2 IS-IS

IPv4: IS-IS routes

R6#show ip route isis

Codes: L - local, C - connected, S - static, R - RIP, M - mobile, B - BGP

D - EIGRP, EX - EIGRP external, O - OSPF, IA - OSPF inter area

N1 - OSPF NSSA external type 1, N2 - OSPF NSSA external type 2

E1 - OSPF external type 1, E2 - OSPF external type 2

i - IS-IS, su - IS-IS summary, L1 - IS-IS level-1, L2 - IS-IS level-2

ia - IS-IS inter area, * - candidate default, U - per-user static route

o - ODR, P - periodic downloaded static route, H - NHRP, l - LISP

a - application route

+ - replicated route, % - next hop override, p - overrides from PfR

Gateway of last resort is not set

10.0.0.0/8 is variably subnetted, 7 subnets, 2 masks

i L2     10.10.1.1/32 [115/10] via 192.168.5.1, 00:30:12, GigabitEthernet0/0/0

i L2     10.10.2.0/24 [115/10] via 192.168.5.1, 1d00h, GigabitEthernet0/0/0

i L2     10.10.3.0/24 [115/10] via 192.168.5.1, 1d00h, GigabitEthernet0/0/0

i L2     10.10.4.0/24 [115/10] via 192.168.5.1, 1d00h, GigabitEthernet0/0/0

i L1     10.10.5.0/24 [115/20] via 192.168.5.1, 1d01h, GigabitEthernet0/0/0

i L2  192.168.1.0/24 [115/10] via 192.168.5.1, 00:30:43, GigabitEthernet0/0/0

i L2  192.168.2.0/24 [115/10] via 192.168.5.1, 1d00h, GigabitEthernet0/0/0

i L2  192.168.3.0/24 [115/10] via 192.168.5.1, 1d00h, GigabitEthernet0/0/0

i L2  192.168.4.0/24 [115/10] via 192.168.5.1, 1d00h, GigabitEthernet0/0/0

IPv6: IS-IS routes

R6#show ipv6 route isis

IPv6 Routing Table - default - 14 entries

Codes: C - Connected, L - Local, S - Static, U - Per-user Static route

B - BGP, R - RIP, I1 - ISIS L1, I2 - ISIS L2

IA - ISIS interarea, IS - ISIS summary, D - EIGRP, EX - EIGRP external

ND - ND Default, NDp - ND Prefix, DCE - Destination, NDr - Redirect

O - OSPF Intra, OI - OSPF Inter, OE1 - OSPF ext 1, OE2 - OSPF ext 2

ON1 - OSPF NSSA ext 1, ON2 - OSPF NSSA ext 2, a - Application

I2  2001:DB8:ACAD:1::/64 [115/10]

via FE80::1, GigabitEthernet0/0/0

I2  2001:DB8:ACAD:2::/64 [115/10]

via FE80::1, GigabitEthernet0/0/0

I2  2001:DB8:ACAD:3::/64 [115/10]

via FE80::1, GigabitEthernet0/0/0

I2  2001:DB8:ACAD:4::/64 [115/10]

via FE80::1, GigabitEthernet0/0/0

I2  2001:DB8:ACAD:11::1/128 [115/10]

via FE80::1, GigabitEthernet0/0/0

I2  2001:DB8:ACAD:12::/64 [115/10]

via FE80::1, GigabitEthernet0/0/0

I2  2001:DB8:ACAD:13::/64 [115/10]

via FE80::1, GigabitEthernet0/0/0

I2  2001:DB8:ACAD:14::/64 [115/10]

via FE80::1, GigabitEthernet0/0/0

I1  2001:DB8:ACAD:15::/64 [115/20]

via FE80::1, GigabitEthernet0/0/0

We have proven IS-IS has been properly configured through showing the Connectionless Network Service (CLNS) neighbors and IS-IS routes. The CLNS neighbors show command shows that the two routes in the IS-IS AS have established adjacency for both IPv4 and IPv6. From the IS-IS routes shown, we see level-1 routes which are within the IS-IS AS proving that IS-IS has been properly configured. Not only that the level-2 routes show that the routes redistributed from BGP have been properly learned and configured. Therefore, IS-IS has been properly established for both IPv4 and IPv6.

Proof of OSPF

IPv4: OSPF neighbors

R1#show ip ospf neighbor

Neighbor ID     Pri   State           Dead Time   Address         Interface

2.2.2.2           1   FULL/DR         00:00:31    192.168.1.2     GigabitEthernet0/0/1

IPv4: OSPF database

R2#show ip ospf database

OSPF Router with ID (2.2.2.2) (Process ID 1)

Router Link States (Area 0)

Link ID         ADV Router      Age         Seq#       Checksum Link count

1.1.1.1         1.1.1.1         983         0x8000014B 0x00FD09 1

2.2.2.2         2.2.2.2         974         0x800000B7 0x00EFA0 1

Net Link States (Area 0)

Link ID         ADV Router      Age         Seq#       Checksum

192.168.1.2     2.2.2.2         990         0x800000B0 0x00A960

Summary Net Link States (Area 0)

Link ID         ADV Router      Age         Seq#       Checksum

10.10.1.1       1.1.1.1         1031        0x800000B0 0x00066C

10.10.2.1       2.2.2.2         1030        0x80000132 0x00D614

Router Link States (Area 1)

Link ID         ADV Router      Age         Seq#       Checksum Link count

2.2.2.2         2.2.2.2         1030        0x800000B2 0x00BA9B 1

Summary Net Link States (Area 1)

Link ID         ADV Router      Age         Seq#       Checksum

10.10.1.1       2.2.2.2         990         0x80000001 0x0051CB

192.168.1.0     2.2.2.2         1030        0x800000B0 0x003BDE

Type-5 AS External Link States

Link ID         ADV Router      Age         Seq#       Checksum Tag

10.10.3.0       2.2.2.2         145         0x800000B0 0x00D328 65002

10.10.4.0       2.2.2.2         145         0x80000107 0x00198A 65002

10.10.5.0       2.2.2.2         402         0x8000002C 0x00C6B7 65002

10.10.6.0       2.2.2.2         402         0x8000002C 0x00BBC1 65002

192.168.2.0     2.2.2.2         145         0x80000132 0x007E7C 0

192.168.3.0     2.2.2.2         145         0x80000132 0x00170D 65002

192.168.4.0     2.2.2.2         402         0x8000002C 0x001B0F 65002

192.168.5.0     2.2.2.2         402         0x8000002C 0x001019 65002

IPv4: OSPF routes

R2#show ip route ospf

Codes: L - local, C - connected, S - static, R - RIP, M - mobile, B - BGP

D - EIGRP, EX - EIGRP external, O - OSPF, IA - OSPF inter area

N1 - OSPF NSSA external type 1, N2 - OSPF NSSA external type 2

E1 - OSPF external type 1, E2 - OSPF external type 2

i - IS-IS, su - IS-IS summary, L1 - IS-IS level-1, L2 - IS-IS level-2

ia - IS-IS inter area, * - candidate default, U - per-user static route

o - ODR, P - periodic downloaded static route, H - NHRP, l - LISP

a - application route

+ - replicated route, % - next hop override, p - overrides from PfR

Gateway of last resort is not set

10.0.0.0/8 is variably subnetted, 7 subnets, 2 masks

O IA     10.10.1.1/32 [110/2] via 192.168.1.1, 00:16:59, GigabitEthernet0/0/0

IPv6: OSPF neighbors

R1#show ipv6 ospf neighbor

OSPFv3 Router with ID (1.1.1.1) (Process ID 1)

Neighbor ID     Pri   State           Dead Time   Interface ID    Interface

2.2.2.2           1   FULL/DR         00:00:32    6               GigabitEthernet0/0/1

IPv6: OSPF database

R2#show ipv6 ospf database

OSPFv3 Router with ID (2.2.2.2) (Process ID 1)

Router Link States (Area 0)

ADV Router       Age         Seq#        Fragment ID  Link count  Bits

1.1.1.1         1267        0x8000014C  0            1           B

2.2.2.2         1539        0x800000BC  0            1           B E

Net Link States (Area 0)

ADV Router       Age         Seq#        Link ID    Rtr count

2.2.2.2         1556        0x80000001  6          2

Inter Area Prefix Link States (Area 0)

ADV Router       Age         Seq#        Prefix

1.1.1.1         1267        0x800000B0  2001:DB8:ACAD:11::1/128

2.2.2.2         1130        0x800000B1  2001:DB8:ACAD:12::1/128

Link (Type-8) Link States (Area 0)

ADV Router       Age         Seq#        Link ID    Interface

1.1.1.1         1267        0x800000B2  7          Gi0/0/0

2.2.2.2         1130        0x800000B3  6          Gi0/0/0

Intra Area Prefix Link States (Area 0)

ADV Router       Age         Seq#        Link ID    Ref-lstype  Ref-LSID

2.2.2.2         1556        0x80000001  6144       0x2002      6

Router Link States (Area 1)

ADV Router       Age         Seq#        Fragment ID  Link count  Bits

2.2.2.2         1596        0x800000B3  0            0           B E

Inter Area Prefix Link States (Area 1)

ADV Router       Age         Seq#        Prefix

2.2.2.2         1130        0x800000B0  2001:DB8:ACAD:1::/64

2.2.2.2         1556        0x80000001  2001:DB8:ACAD:11::1/128

Intra Area Prefix Link States (Area 1)

ADV Router       Age         Seq#        Link ID    Ref-lstype  Ref-LSID

2.2.2.2         1130        0x800000AF  0          0x2001      0

Type-5 AS External Link States

ADV Router       Age         Seq#        Prefix

2.2.2.2         625         0x80000002  2001:DB8:ACAD:3::/64

2.2.2.2         625         0x80000002  2001:DB8:ACAD:5::/64

2.2.2.2         625         0x80000002  2001:DB8:ACAD:13::/64

2.2.2.2         625         0x80000002  2001:DB8:ACAD:14::/64

2.2.2.2         625         0x80000002  2001:DB8:ACAD:15::/64

2.2.2.2         625         0x80000002  2001:DB8:ACAD:16::/64

2.2.2.2         368         0x80000002  2001:DB8:ACAD:4::/64

IPv6: OSPF routes

R2#show ipv6 route ospf

IPv6 Routing Table - default - 15 entries

Codes: C - Connected, L - Local, S - Static, U - Per-user Static route

B - BGP, R - RIP, I1 - ISIS L1, I2 - ISIS L2

IA - ISIS interarea, IS - ISIS summary, D - EIGRP, EX - EIGRP external

ND - ND Default, NDp - ND Prefix, DCE - Destination, NDr - Redirect

O - OSPF Intra, OI - OSPF Inter, OE1 - OSPF ext 1, OE2 - OSPF ext 2

ON1 - OSPF NSSA ext 1, ON2 - OSPF NSSA ext 2, a - Application

OI  2001:DB8:ACAD:11::1/128 [110/1]

via FE80::1, GigabitEthernet0/0/0

The show command for the OSPF neighbors, OSPF database, and OSPF routes prove that OSPF has been properly established for both IPv4 and IPv6. The OSPF neighbor database shows that the two routers in the OSPF AS have properly established adjacency for both IPv4 and IPv6. Not only that, the OSPF database shows routes to all of the network proving that the routes redistributed from BGP have been properly learned. And finally, the routing tables show that multiarea OSPF has been configured as R2 can see the far loopback interface of R1 which is in another area. Therefore, multiarea OSPF has been properly configured for both IPv4 and IPv6.

Problems

EIGRP redistribution

Redistribution of routes into EIGRP requires more than redistribution into IS-IS or OSPF. OSPF and IS-IS use cost as the only metric (link cost specifically for IS-IS) while EIGRP uses a completely different set of metrics (bandwidth, delay, reliability, load, MTU). Because the cost does not have a direct translation into EIGRP metrics, a simple redistribution command, redistribute bgp 65001 include-connected, into EIGRP will not work. Without proper redistribution, the EIGRP AS will fail to learn routes to the other ASes and stop packets travelling across the network. This summarizes the problem that we faced redistributing BGP routes into EIGRP. Our solution was to simply set EIGRP metrics manually. We set the metrics to the default as follows: redistribute bgp 65001 include-connected 10000 100 255 1 1500. This solved the problem and allowed the AS using EIGRP to learn BGP routes.

IS-IS Layers and redistribution

ISIS uses two general systems of categorization level 1 & level 2. Our initial configuration put R5 in level 1 and R6 in level 2. This caused a major problem. When we redistributed our IS-IS routes using the command redistribute isis level-1, it didn’t redistribute any IS-IS routes. The solution was to understand that level 2 manages interarea routing rather than a separate area like multiarea OSPF. But with our second configuration we included the ethernet connection between R5 and R6 in level 1 but put loopback interfaced in level 2. This caused the routes to the loopback interfaced to be left out during redistribution. Only after we put all the interfaces into level 1 did redistribution work. The reason why it works was because all the interfaces and routers were within the same area and because we only redistributed level 1 routes, level 2 routes would be excluded.

Incorrect ASN

When implementing BGP over IPv6, we made a simple but significant error. In order to establish a BGP peer adjacency, the ASN and IPv6 address of the neighbor must be manually added. But on R4, we implemented the following command into our configuration: neighbor 2001:DB8:ACAD:4::2 remote-as 65003.  And on R5 we implemented neighbor 2001:DB8:ACAD:4::1 remote-as 65001. After implementation, the routers actively sought to open a session with each other. But because we implemented the ASN wrong, each session would suddenly close, failing to establish adjacency. The result was that our PuTTY terminal filled up with debug messages and failed connection & redistribution. We solved this issue by shutting down the interfaces and correctly implementing the proper ASNs. No debug messages popped up afterwards.

Conclusion

BGP is a useful EGP protocol that can be used to connect a handful of ASes using different IGP protocols together. Not only is BGP compatible with a multitude of IGPs, it can also be actively used to manage traffic between ASes. Using ASNs, weights, and other similar tools, users can manage and manipulate the flow of traffic between ASes and mitigate attacks using tools like route maps to block traffic from unknown or suspicious devices.